| MECC 2026 SPECIAL SESSION • HANDS-ON ACTIVITY Tune the Pendulum Explore P, PI, PD, and PID control through hands-on gain tuning |  |  |  |  |
| --- | --- | --- | --- | --- |
| 15 MINUTES | WORK IN PAIRS | YOUR DESIGN VARIABLES: Kp, Ki, Kd |  |  |

Why? A controller’s performance depends strongly on how it is tuned. In this activity, you will change Kp, Ki, and Kd on a real pendulum, observe how each term changes the measured response, and use your observations to design a controller that tracks the target.

By the end you will be able to:

- Predict how changes in Kp, Ki, and Kd will affect the pendulum response.

- Compare your prediction with measured overshoot, steady-state error, and oscillation.

- Design and justify a set of PID gains that produces an acceptable tracking response.

MEET THE MODEL: WHAT YOU WILL USE

| Guide to the provided Simulink model. The gold blocks (5, 6, 8) are the ones you use. | The model is already built. Blocks 1–4 and 7 read the encoder, filter the signals, and run the PID controller, so leave them alone. 5 SET TARGET Set the two targets to +45° and −45° with a 2 s hold at each (period 4 s). 6 PID TUNER Move the sliders or type values for Kp, Ki, Kd. Start with Ki = 0 and Kd = 0. 8 GRAPHS Open the Target vs. Angle (deg) scope. Read overshoot and steady-state error here. Target: +45° for 2 s, then −45° for 2 s |
| --- | --- |

REFERENCE: HOW EACH PID GAIN AFFECTS THE RESPONSE

| Increase this gain | Rise time | Overshoot | Settling time | Steady-state error | Practical note |
| --- | --- | --- | --- | --- | --- |
| Kp | ▼ Decreases | ▲ Increases | ● Small change | ▼ Decreases | Faster response; too large causes oscillation and instability. |
| Ki | ▼ Decreases | ▲ Increases | ▲ Increases | ✔ Eliminates | Removes steady offset; too large causes overshoot and slow oscillation. |
| Kd | ● Small change | ▼ Decreases | ▼ Decreases | ● No change | Adds damping; too large amplifies encoder noise and makes the motor buzz. |

▼ helps   ▲ hurts   ● little effect.   These are typical trends when one gain changes and the others stay fixed. The gains interact, so use them to make predictions, not as guarantees.

GROUND RULES - Change one gain at a time so you can link each change in the response to its cause. - Predict first. Write your expectation in the Observations column, then run and compare. - Safety: keep the supply at or below 12 V. If the pendulum oscillates violently, stop the model and lower the last gain you raised.

RECORD YOUR DESIGN ATTEMPTS

Starting gains from your facilitator: Kp = ______ Ki = ______ Kd = ______ How to measure: Overshoot (%) = peak beyond the target ÷ 90° × 100 (the step between targets is 90°). Steady-state error (°) = target minus measured angle at the end of the 2 s hold.

| 1 P CONTROLLER Set Ki = 0 and Kd = 0. Try a small, a medium, and a large Kp. |  |  |  |  |
| --- | --- | --- | --- | --- |
| Trial | Kp | Overshoot (%) | Steady-state error (°) | Observations: prediction, rise, oscillation, noise |
| 1 |  |  |  |  |
| 2 |  |  |  |  |
| 3 |  |  |  |  |
| Reflect: How did increasing Kp change overshoot and steady-state error? __________________________________ |  |  |  |  |

| 2 PI CONTROLLER Keep your best Kp. Add Ki and increase it gradually. |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Trial | Kp | Ki | Overshoot (%) | Steady-state error (°) | Observations: prediction, rise, oscillation, noise |
| 1 |  |  |  |  |  |
| 2 |  |  |  |  |  |
| 3 |  |  |  |  |  |
| Reflect: What did Ki do to steady-state error, and what did it cost? __________________________________ |  |  |  |  |  |

| 3 PD CONTROLLER Set Ki = 0. Keep your best Kp. Add Kd and increase it gradually. |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Trial | Kp | Kd | Overshoot (%) | Steady-state error (°) | Observations: prediction, rise, oscillation, noise |
| 1 |  |  |  |  |  |
| 2 |  |  |  |  |  |
| 3 |  |  |  |  |  |
| Reflect: What did Kd do to overshoot and oscillation? Did you notice noise? __________________________________ |  |  |  |  |  |

| 4 PID CONTROLLER Use all three gains to get your best overall response. Aim for the targets below. |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Trial | Kp | Ki | Kd | Overshoot (%) | Steady-state error (°) | Observations: prediction, rise, oscillation, noise |
| 1 |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |
| 4 |  |  |  |  |  |  |
| Targets: overshoot below 10%, steady-state error below 2°, and no sustained oscillation at either hold. |  |  |  |  |  |  |
| My final gains: Kp = ________ Ki = ________ Kd = ________ |  |  |  |  |  |  |

WRAP-UP: DISCUSS WITH YOUR PARTNER  (1 MIN)

1   Which gain changed the response the most? What evidence in your tables supports this?

2   Describe one tradeoff you saw, where improving one characteristic made another worse.

3   How would you use prediction, observation, and evidence to tune a different system?